Choosing the right VHF data link technology for commercial aviation air traffic services

by

Melwyn Savio Pereira

Bachelor of Science Computer Science

California Polytechnic State University

San Luis Obispo, California

June 1995

A thesis submitted to the

Faculty of the Graduate School of Engineering at the

University of Colorado in partial fulfillment

Of the requirement for the degree of

Master of Science

Telecommunications

May 2002

This thesis for the Master of Science degree by

Melwyn Savio Pereira

Has been approved for the Program in Telecommunications

By

_______________________________________

# Harvey Gates

_______________________________________

# Frank Barnes

_______________________________________

# Scott Savage

_______________________________________

# Gerald Mitchell

The final copy of this thesis has been examined by the signatories, and we find both the content and the form meet acceptable presentation standards of scholarly work in the above mentioned discipline.

# Abstract

Choosing the right VHF data link technology for commercial aviation air traffic services.

Thesis directed by Dr. Harvey Gates.

The Very High Frequency (VHF) Amplitude Modulation (AM) channel used for voice communications among air traffic control and commercial airline pilots is congested. This contributes to National Airspace System (NAS) system inefficiencies such as aircraft delays, diminishing airline profits and most importantly a compromise in the safety of passengers and flight crew. Although there are elaborate data linking designs in progress to address these problems through concepts such as Free Flight, Satellite-based Air Traffic Management Systems, etc., it could be years before a data link is implemented.  Because a data linking solution is critical to the safety and efficiency of airline operations, VHF data link (VDL) technology should be immediately implemented to provide effective and reliable communications between pilots and air traffic controllers.

The three competing digital data link technologies are VDL modes 2, 3 & 4. VDL mode 2 and mode 3 are considered to be primarily communications solutions while VDL mode 4 also offers surveillance capabilities. First, VDL mode 2 provides a data rate that is ten (10) times faster than the current ACARS system; however, it uses the carrier sense multiple access (CDMA) protocol to access the channel. Second, VDL mode 3 (NEXCOM) has the ability to transmit and receive digital voice and data through the time division multiple access protocol (TDMA). However, if it were to transmit data exclusively, it would use the CSMA protocol. Lastly, VDL mode 4 provides digital data as well as provides a built-in surveillance capability. It uses the self-organized time division multiple access (STDMA) protocol to access the media, which is more efficient than CSMA for channel access. Also, it’s surveillance capabilities allow future air traffic initiatives such as Free Flight to be seamlessly accommodated. Consequently, VDL mode 4’s should be considered as the right data link technology for commercial aviation air traffic services.

Dedication

This thesis is dedicated to the memory of those pilots and families of September 11, 2001 who paid the ultimately price by surrendering their lives for our freedom; and to my wife and friend Ramona who supported me with her love and patience through this long and windy academic pursuit by gently encouraging me and giving me distance from reality by running our house-hold. Lastly, to my Lord for his providence and divine guidance.

Acknowledgements

I can never adequately express my gratitude to everyone who assisted me on my thesis, but I would like to mention those people who provided extraordinary guidance.  From the University of Colorado, my advisor Professor Harvey Gates who gave generously of his time and provided a vision to combine my interests in aviation and telecommunications. Professors Gerald Mitchell, Frank Barnes and Scott Savage who provided constructive feedback and gave generously of their time and talent.

The Department of Commerce’s, Chris Behm, Dr. Margaret Luebs and Dr. Rob Stafford who provided feedback and editorial comments on specific sections of this document. Mr. Evan Darby whose outstanding public service should be a model for all to follow at the Federal Aviation Administration William J. Hughes Technical Center. His correspondence over the past four months has provided me with a real-time understanding of the data link communication initiatives.  Also, Mr. Theo Hendriks, Mr. Daniel Daems and Mr. Luca Dell’Orto from Eurocontrol offered valuable insight into the European data link initiatives.

Lastly, but not least, the Pereira and Dancel families who gave their love and support in the most challenging times of my life who made me realize the value of an education.

# TABLE OF CONTENTS

Chapter One – Introduction

Air transportations impact to the United States economy…………………………1

Figure 1-1: United States Air Travel Demand 1940 – 2000.…………………………2

Figure 1-2: Airline price and demand 1940 to 2000……………………………………4

Figure 1-3:”Delays greater than 15 minutes caused by

weather 1985-2000”……….……………………………………………………5

Figure 1-4: Runway incursions 1993-1998……………………………………………………7

Figure 1-5: Runway incursions 1993-1998……………………………………………………8

Economics of congestion………………………………………………………………………………9

Engineering solution…………………………………………………………………….…9

Supply side solution………………………………………………………………………10

Demand side solution……………………………………………………………………11

Purpose of the Study………………………………………………………………………………….11

Scope of the Study…………………………………………………………………………………….12

Methodology and Arrangement of the Thesis……………………………………………12

# Chapter Two - Historical Background

Aeronautical Radio Navigation Spectrum………………………………………………….15

Airspace……………………………………………………………………………………………….…….15

Figure 2-1 Sector Alpha with frequency A………………………………………15

Figure 2-2 Sector Alpha with frequency A & B……………………………….16

Figure 2-3 Atlanta airspace frequency allocations…………………………17

ACARS Infrastructure………………………………………………………………………………….18

Table 2-1 Air to Ground Communications Architecture…………………19

ACARS Applications………………………………………………………………………….19

Table 2-2 OOOI message example………………………………………………….20

Figure 2-4 METAR weather example……………………………………………….21

ACARS Limitations………………………………………………………………………………………22

Applications………………………………………………………………………………………………22

Sub-network………………………………………………………………………………….22

Data-link layer………………………………………………………………………………23

The change in air transport traffic……………………………………………………………24

The VHF Digital Data Link………………………………………………………………………….25

Chapter Three – Technology

Table 3-1 OSI model……………………………………………………………………………………29

Table 3-2 OSI model referenced with VDL Technology…………………………….30

Physical Layer……………………………………………………………………………………….……30

Data Link Layer………………………………………………………………………………………….30

Network Layer……………………………………………………………………………………………31

VDL mode 2 detailed analysis……………………………………………………………………32

Feasibility analysis…………………………………………………………………………33

VDL mode 3 detailed analysis……………………………………………………………………34

Feasibility analysis…………………………………………………………………………35

VDL mode 4 detailed analysis……………………………………………………………………36

Feasibility analysis…………………………………………………………………………37

VDL modes 2, 3, 4 comparison tables……………………………………………………….38

Table 3-3 Physical characteristics…………………………………..………….38

Table 3-4 Data communications characteristics I. ………………………38

Table 3-5 Data communications characteristics II. …………………….39

Capabilities Analysis for VDL modes 2, 3 and 4 ……………………………………….39

VDL mode 3……………………………………………………………………………………39

Figure 3-1 TDMA Frame…………………………………………………………..……40

Figure 3-2 TDMA Communications Model………………………………………40

VDL mode 2……………………………………………………………………………………42

VDL mode 4……………………………………………………………………………………42

Chapter Four – Economics

Air Traffic Services…………………………………………………………………………………….44

Table 4-1 Estimated ground base station costs…………………………………………45

Table 4-2 Estimated aircraft equipment costs………………………………………….46

Figure 4-1 VDL Categorizations………………………………………………………………….48

Figure 4-2 VDL Technology time line…………………………………………………………49

Table 4-3 Estimated Cost……………………………………………………………………………50

Table 4-4 Application and Data Link technology requirement…………………51

Chapter Five – Aviation Public Policy

VDL mode 2, 3 and 4 spectrum usage……………………………………………………….55

Inhibitors to VDL Technology…………………………………………………………………….56

Air traffic control regulation…………………………………………………………56

Airline communications service monopoly……………………………………56

Promoters of VDL Technology……………………………………………………………………58

Artificial incentives for airliners………………………………………………….58

Deregulation of the air traffic control sector………………………………59

Taxes…………………………………………………………………………………………….60

National Security Interests……………………………………………………………61

Chapter Six - Findings

Table 6-1 Results Comparison……………………………………………………………………64

Airwave congestion………………………………………………………………….…………………65

Runway incursions………………………………………………………………………………………65

Weather: Terminal and En-route………………………………………………………………66

Figure 6-1 Typical daytime U.S. airspace traffic………………………………………66

Future Research……………………………………………………………………….……………….68

Bibliography……………………………………………………………………………………………..71

# TABLES

Table 2-1 Air to Ground Communications Architecture …….…………….………19

Table 2-2 OOOI message example………………………………………………….…………20

Table 3-1 OSI model……………………………………………………………….……….…………29

Table 3-2 OSI model referenced with VDL Technology………….……………...30

Table 3-3 Physical characteristics……………………………………..…….………………38

Table 3-4 Data communication characteristics I. ……………………..…………….38

Table 3-5 Data communication characteristics II. ……………….………………….39

Table 4-1 Estimated ground base station costs…………………………………………45

Table 4-2 Estimated aircraft equipment costs………………………………………….45

Table 4-3 Estimated costs……………………………………………..………………………….49

Table 4-4 Application and Data Link technology requirement..………………50

Table 6-1 Results Comparison……………………………………………………………………62

# FIGURES

Figure 1-1: U.S. Air Travel Demand…………………………………………………………….2

Figure 1-2: Airline price and demand 1940 to 2000……………………………………4

Figure 1-3: “Delays greater than 15 minutes caused by

weather 1985-2000”…………………………………………………………………7

Figure 1-4: Runway incursions 1993-1998……………………………………………..……7

Figure 1-5: Runway incursions 1993-1998…………………………………………..….….8

Figure 2-1 Sector Alpha with frequency A…………………………………………………15

Figure 2-2 Sector Alpha with frequency A & B…………………………………….……16

Figure 2-3 Atlanta airspace frequency allocations……………………………..……17

Figure 2-4 METAR weather examples………………………………………………….…….21

Figure 3-1 TDMA Frame……………………………………………………………………….…….40

Figure 3-2 TDMA Communications Model………………………………………………….40

Figure 4-1 VDL Categorizations………………………………………………………………….47

Figure 4-2 VDL Technology Time Line……………………………………………………….48

Figure 6-1 Typical daytime U.S. airspace traffic………………………………………65